EE Architecture Design

JSON Generator

Functional & Technical Specification

VS Code-like UI • Full System Tree Editing • C Framework-Compatible JSON Export

Author : Anadack Temtching Dassi
Date: 11 June 2026
Version: 1.0

# 1. Document Purpose

This document specifies the expected behavior, user interface, data model, validation rules, library integration, and export format of the EE Composer JSON Generator. The generator is a browser-based tool used to create, edit, validate, import, and export JSON models that are directly importable by the EE_Composer C framework.

The generator must support full editing of an electrical/electronic system hierarchy, including systems, components, sensors, actuators, connectors, pins, and signals. The exported JSON must be importable by the C framework without manual correction.

System
└── Component[]
    ├── Sensor[]
    │   ├── Connector[]
    │   ├── Pin[]
    │   └── Signal[]
    └── Actuator[]
        ├── Connector[]
        ├── Pin[]
        └── Signal[]

# 2. Main Objectives

Provide a Visual Studio Code-like user interface.

Use a resizable multi-panel layout.

Display a library explorer on the left.

Display a structured object editor in the center.

Display a live JSON view on the right.

Display a terminal/log panel at the bottom.

Allow full editing of System, Component, Sensor, Actuator, Connector, Pin, and Signal objects.

Allow adding and removing objects in the system hierarchy.

Allow adding existing objects from the library into a compatible parent object.

Make all IO-to-ECU mapping fields mandatory.

Export JSON files directly importable by the EE_Composer C framework.

# 3. UI Layout Specification

## 3.1 Global Layout

The UI shall follow a Visual Studio Code-style layout:

┌────────────────────────────────────────────────────────────────────────────┐
│ Top Toolbar: New | Import | Export | Validate | Save | JSON View          │
├───────────────────┬────────────────────────────────┬─────────────────────┤
│ Library Explorer  │ Object Editor                  │ JSON View           │
│                   │                                │                     │
│ systems/          │ Editable form for selected     │ Current object JSON │
│ components/       │ object                         │ C import JSON       │
│ sensors/          │                                │                     │
│ actuators/        │                                │                     │
│ connectors/       │                                │                     │
│ pins/             │                                │                     │
│ signals/          │                                │                     │
├───────────────────┴────────────────────────────────┴─────────────────────┤
│ Terminal / Logs / Validation / Modification History                       │
└────────────────────────────────────────────────────────────────────────────┘

## 3.2 Resizable Views

Left library panel shall be resizable.

Center editor panel shall be resizable.

Right JSON panel shall be resizable.

Bottom terminal panel shall be vertically resizable.

The current panel sizes should persist during the session.

# 4. Main UI Panels

## 4.1 Top Toolbar

The top toolbar shall provide the following actions:

New System

New Component

New Sensor

New Actuator

New Connector

New Pin

New Signal

Import JSON

Import Library Folder

Export C-Compatible JSON

Export Selected Object JSON

Validate

Save

Save As

Copy JSON

Toggle JSON View

The toolbar shall display the current model status:

VALID | INVALID | DIRTY | SAVED | EXPORT BLOCKED

## 4.2 Library Explorer

The left panel shall display objects available in the library.

library/
├── systems/
├── components/
├── sensors/
├── actuators/
├── connectors/
├── pins/
└── signals/

Open a JSON object from the library.

Add a library object to the currently selected compatible parent.

Clone a library object into the current system.

Remove an object from the current system.

Optionally delete an object from the library as a separate explicit action.

Important rule: adding an object from the library into a system shall create a local copy, not a live reference.

## 4.3 Center Object Editor

The center panel shall display a form for the currently selected object. All fields belonging to the selected object structure shall be visible and editable.

System

Component

Sensor

Actuator

Connector

Pin

Signal

The editor shall support text fields, numeric fields, boolean switches, enum dropdowns, array editors, add/remove child actions, required field indicators, and mapping-critical field indicators.

## 4.4 Right JSON View

The right panel shall display JSON for the selected object and shall provide two tabs:

Object JSON

C Import JSON

Object JSON shows the selected object exactly as it exists in the editor state. C Import JSON shows the final normalized JSON that will be exported for the C framework.

## 4.5 Bottom Terminal Panel

The terminal shall display all important actions, modifications, validations, warnings, errors, and exports.

[INFO] Generator started
[LIBRARY] Loaded PressureSensor.json
[ADD] Sensor PressureSensor added to component HydraulicSensors
[EDIT] Pin PIN1_SIGNAL interface_type changed from DIGITAL to ANALOG
[REMOVE] Connector X1 removed from PressureSensor
[VALIDATE] Checking system HydraulicSystem
[OK] Pin 1 references signal PressureSensor_Value
[ERROR] Pin 2 missing electrical_requirement
[BLOCKED] Export cancelled: model is not importable by C framework
[EXPORT] HydraulicSystem.json generated successfully

# 5. Editable Object Hierarchy

## 5.1 System

Editable fields:

type

name

version

system_level

priority

safety

Ref-2X

part_number

location

take_rate

is_mandatory

platforms

brands

description

manufacturer

notes

components

## 5.2 Component

Editable fields:

type

name

part_number

supplier_pn

priority

safety

manufacturer

description

notes

sensors

actuators

## 5.3 Sensor

Editable fields:

name

device_type

priority

safety

part_number

supplier_pn

manufacturer

description

notes

owner_system

owner_architecture

connectors

pins

signals

The value of device_type shall be SENSOR. A sensor shall allow adding and removing Connector, Pin, and Signal objects.

## 5.4 Actuator

Editable fields:

name

device_type

priority

safety

part_number

supplier_pn

manufacturer

description

notes

owner_system

owner_architecture

connectors

pins

signals

The value of device_type shall be ACTUATOR. An actuator shall allow adding and removing Connector, Pin, and Signal objects.

## 5.5 Connector

Editable fields:

name

part_number

family

gender

total_cavities

used_cavities

max_pin_number

sealed

color

mounting

ip_rating

rated_voltage

rated_current

temperature_min

temperature_max

## 5.6 Pin

Editable fields:

number

name

role

interface_type

electrical_requirement

signal

nominal_current

inrush_current

max_voltage

diagnostics_required

safety_relevant

ground_class

required_reset_state

required_sensor_supply

required_sensor_ground

required_supply_voltage

All pin fields used for IO-to-ECU mapping are mandatory. The signal field shall reference an existing signal inside the same sensor or actuator.

## 5.7 Signal

Editable fields:

prefix

name

count

type

interface

interface_type

unit

min

max

min_value

max_value

resolution

scaling

priority

safety

part_number

digital_structure

electrical_requirement

role

description

notes

is_mapped

The generator shall normalize signal fields during export to match the C framework import format.

# 6. Add and Remove Rules

## 6.1 Parent/Child Compatibility Matrix

| Parent | Allowed Children |
| --- | --- |
| System | Component, Sensor, Actuator |
| Component | Sensor, Actuator |
| Sensor | Connector, Pin, Signal |
| Actuator | Connector, Pin, Signal |
| Pin | Signal association only |
| Connector | None |
| Signal | None |

## 6.2 Adding Objects from Library

The selected library object shall be cloned.

The clone shall be inserted into the selected compatible parent.

The clone shall become part of the current system model.

The original library object shall remain unchanged.

## 6.3 Removing Objects

Removing an object from the current system shall remove only the local instance. Deleting from the library shall be a separate explicit action.

# 7. Data Model

## 7.1 Internal UI Model

{
  "type": "system",
  "name": "HydraulicSystem",
  "components": [
    {
      "type": "component",
      "name": "HydraulicComponent",
      "sensors": [],
      "actuators": []
    }
  ]
}

## 7.2 C Framework Export Model

The exported JSON must be compatible with the C framework. For system import, the generator shall export:

{
  "type": "system",
  "name": "HydraulicSystem",
  "version": "1.0",
  "system_level": "SL2",
  "priority": "HIGH",
  "safety": "ASIL_A",
  "Ref-2X": "",
  "part_number": "SYS-HYD-001",
  "location": "Vehicle",
  "take_rate": 100,
  "is_mandatory": true,
  "platforms": [],
  "brands": [],
  "description": "",
  "manufacturer": "",
  "notes": "",
  "devices": []
}

The generator shall flatten components[].sensors[] and components[].actuators[] into devices[].

# 8. Export Adapter

The export adapter shall transform the internal UI model into the C-compatible JSON model.

For each component in system.components:
    For each sensor in component.sensors:
        Normalize sensor as device_type SENSOR
        Add to devices[]

    For each actuator in component.actuators:
        Normalize actuator as device_type ACTUATOR
        Add to devices[]

Create final system JSON:
    type, name, version, system_level, priority, safety, Ref-2X,
    part_number, location, take_rate, is_mandatory, platforms,
    brands, description, manufacturer, notes, devices[]

# 9. Validation Rules

Validation shall be executed before export. Export shall be blocked if any required field is missing or inconsistent.

## 9.1 System Validation Errors

System.name is empty

System.system_level is empty

System.priority is empty

System.safety is empty

System has no component

System has no device after export flattening

## 9.2 Component Validation Errors

Component.name is empty

Component has no sensor and no actuator

## 9.3 Sensor / Actuator Validation Errors

Device.name is empty

Device.device_type is empty

Device.priority is empty

Device.safety is empty

Device has no connector

Device has no pin

Device has no signal

## 9.4 Connector Validation Errors

Connector.name is empty

Connector.total_cavities is missing

Connector.used_cavities is missing

Connector.max_pin_number is missing

Connector.used_cavities > Connector.total_cavities

Connector.max_pin_number < Connector.used_cavities

## 9.5 Pin Validation Errors

Pin.number is missing

Pin.name is empty

Pin.role is empty

Pin.interface_type is empty

Pin.electrical_requirement is missing

Pin.signal is empty

Pin.signal does not reference an existing signal

Pin.nominal_current is missing

Pin.inrush_current is missing

Pin.max_voltage is missing

Pin.diagnostics_required is missing

Pin.safety_relevant is missing

Pin.ground_class is empty

Pin.required_reset_state is empty

Pin.required_sensor_supply is empty

Pin.required_sensor_ground is empty

Pin.required_supply_voltage is missing

## 9.6 Signal Validation Errors

Signal.prefix or Signal.name is empty

Signal.type is empty

Signal.interface or Signal.interface_type is empty

Signal.unit is empty

Signal.min is missing

Signal.max is missing

Signal.resolution is missing

Signal.scaling is missing

Signal.priority is empty

Signal.safety is empty

Signal.digital_structure is empty

Signal.electrical_requirement is missing

Signal.role is empty

Signal.min > Signal.max

Signal.resolution <= 0

Signal.count <= 0

# 10. Required Enumerations

## Priority

LOW
MEDIUM
HIGH
CRITICAL

## Safety

QM
ASIL_A
ASIL_B
ASIL_C
ASIL_D

## Device Type

SENSOR
ACTUATOR

## Pin Role

INPUT
OUTPUT
SUPPLY
GROUND
COMMUNICATION

## Signal Interface

ANALOG
DIGITAL
PWM
CAN
LIN
SENT
ETHERNET
FREQUENCY
RESISTANCE
CURRENT
FLEXRAY
POWER

## Digital Structure

UNSPECIFIED
PULLUP
PULLDOWN
FLOATING
PUSH_PULL
OPEN_DRAIN

# 11. Import Requirements

The generator shall import System JSON, Component JSON, Sensor JSON, Actuator JSON, Connector JSON, Pin JSON, Signal JSON, and Bundle JSON.

When importing a C-compatible system JSON containing devices[], the generator shall reconstruct an editable internal model.

If component information is unavailable, the generator may create ImportedComponent, ImportedSensors, or ImportedActuators containers.

# 12. Export Requirements

Export selected object JSON.

Export full internal system JSON.

Export C-compatible system JSON.

Export library object JSON.

Copy JSON to clipboard.

Download JSON file.

Block C-compatible export if validation fails.

# 13. Error Handling

All invalid operations shall be blocked with explicit terminal messages.

Warnings shall not block export unless strict mode is enabled.

Errors shall always block C-compatible export.

# 14. Strict Export Mode

All warnings become blocking errors.

All mapping-related fields are mandatory.

JSON export is allowed only if the full system is C-importable.

Unknown enum values are rejected.

Unknown fields may be preserved but shall not be used for export.

# 15. Expected Workflow

Open the generator.

Import or create a new system.

Load the library folder.

Select a component, sensor, actuator, connector, pin, or signal from the library.

Add the selected object to a compatible parent.

Edit object fields in the center editor.

Check JSON in the right panel.

Validate the full system.

Fix all errors.

Export C-compatible system JSON.

Import the exported JSON into the EE_Composer C framework.

# 16. Acceptance Criteria

The UI behaves like a VS Code-style editor.

The user can resize the main panels.

The user can edit the complete system hierarchy down to signals.

The user can add and remove compatible child objects.

The user can add existing library objects into the current system.

Every object field is visible and editable.

Mapping-critical fields are mandatory.

The JSON view updates live.

Validation detects missing or inconsistent data.

Export is blocked when the model is not valid.

The exported system JSON is directly importable by the EE_Composer C framework.

The terminal logs all user actions and validation results.

# 17. Final Design Principle

The generator shall be both a visual system composition tool and a strict C-framework-compatible JSON authoring tool.

The user experience should be flexible for editing, but the export process must be strict.

If the generator exports a system JSON, that JSON must be importable by the EE_Composer C framework without manual correction.